S5863-24H2QC

L3 Managed Ethernet Switch

# Product Overview

The S5863-24H2QC switch is a high-performance Layer 3 switch that supports multi-rate access at 1G/2.5G/5G/10G and offers 40G and 100G uplink core routing. This switch is designed to meet the high-density and high-bandwidth requirements of next-generation enterprise networks, data centers, and metropolitan area networks. It is also suitable for telecom network operators, as well as the aggregation or access layers in campus Ethernet networks, and can serve as a connection layer in data centers or as the core layer for small and medium-sized enterprises, providing extensive server access solutions.

The S5863-24H2QC offers a comprehensive set of service features, including Layer 2, Layer 3, VXLAN, OAM/APS, and others, along with robust security and flow control features, hardware-based NetFlow, and high-precision clock capabilities. The switch supports elastic stacking, allowing multiple devices to be virtualized into a single logical entity. This feature facilitates port expansion and switching capacity increases while enabling unified management, upgrades, and maintenance of multiple devices.

Designed to handle large data traffic and ensure non-blocking transmission in data centers, the S5863-24H2QC provides significant cache capacity and supports an advanced cache scheduling mechanism to optimize the use of this capacity. It is versatile enough for use in network operator environments, campus Ethernet aggregation or access layers, data center access layers, and as a core layer in small and medium-sized enterprises, offering rich server access solutions for data centers.

# Device Brief Information

|  | S5863-24H2QC |
| --- | --- |
|  | - 24 * 1G/2.5G/5G/10G Base-T - 2*40G/100G QSFP28 - 1* power module supply - 4* modular Fans - Size: 440X320X44mm 19” Rack-mount - AC: 100～240V; 47/63Hz |

Key Features

Equipment Stability

- Support IPv4/IPv6 dual protocol stack platform based on Linux operating system

- Support multiple link redundancy and network redundancy protocols such as STP / RSTP / MSTP / ERPS / LACP

Carrier-level equipment stability

- Based on Linux operating system, support IPv4/IPv6 dual protocol stack platform

- Support multiple link redundancy and network redundancy protocols such as STP/RSTP/MSTP/ERPS/LACP

Stackable

- Support Combine the processing power of multiple switches into a single logical device.

- Support When a switch in the stack system fails, traffic can be automatically switched to other switches, network availability and redundancy .

- Support Stacking simplifies the management process. Administrators only need to manage one logical device.

Perfect safety mechanism

- Support ACL security filtering mechanism, which can provide security control functions based on MAC, IP, L4 port

- Support ARP attack automatic protection and user blocking functions

- Support DHCP attack automatic protection based on Mac Address and user blocking functions

- Support DHCP-snooping/IP-source-Guard/802.1X and other security features

- Support link protection functions such as BFD, FlexLink, LACP

- Support remote loop detection function

- Support remote user authentication based on Tacacs+, Radius, and Local user authentication

Powerful QoS capabilities

- Support port speed and flow speed limit function

- Support classification based on service flow and Qos flow control function

- Support queue scheduling algorithms such as SP/WRR/SP+WRR

- Support port-based and service flow-based mirroring functions

Enhanced multi-service capabilities

- Support DHCP Server and DHCP Relay

Rich routing protocol

- Support static route

- Support dynamic route such as RIP, OSPF, BGP,

- Support IPV4/IPV6 dual protocol stack, support RIPng、OSPFv3

Simple and easy-to-use network management function

- Support CLI based on RS232 serial port, Telnet and SSHv2

- Support WEB-based configure operation management, support SNMP V1/V2/V3

- Support remote upgrade or equipment through FTP and TFTP

Hardware Specifications

| Attributes | S5863-24H2QC |
| --- | --- |
| Port | 24*1G/2.5G/5G/10G Base-T; 2 * 40G/100G QSFP28 |
| Exchange Capacity | 880Gbps |
| Packet Forwarding Rate | 654Mpps |
| Memory and Storage | Memory: 1GB; Storage: 8GB |
| MAC Address | 32K |
| Packet Buffer | 72Mbit |
| Power | AC: 100～240V; 47/63Hz |
| Power Consumption | Idle ≤33W Full load ≤60W |
| Weight | ≤4.4kg |
| Dimensions (W*H* D) | 440X320X44 (mm*mm*mm) |
| Environment | Working Temperature：-10℃～50℃ Storge Temperature：-40℃～70℃ Relative Humidity: 5%~95%, non-condensing |

Panel view

LED indicator

| LEDs/Buttons | Color | Function | Description |  |
| --- | --- | --- | --- | --- |
| PWR | Green | Power status | ON | The AC/DC power module is normal (right) |
|  |  |  | OFF | The AC/DC power module faulty or unavailable |
| ALARM | Red | Port status | ON | Critical alarms are reported for the card. |
|  |  |  | OFF | No critical alarm is reported for the card. |
| SYS | Green | System running status | Green OFF | The device is not running or power off. |
|  |  |  | Green Flash | The device is running normally. |
| MGMT | Green | Port status | ON | The link is properly connected (link up) and in action |
|  |  |  | OFF | The link is interrupted (link down). |
| STK | Green | Port status | ON | Enable Stack |
|  |  |  | OFF | Disable Stack |
| USB | Green | Power status | ON | Insert flash (Fat32) |
|  |  |  | OFF | NO Insert flash (Fat32) |
| 1G/2.5G/5G/10G | Green/ Orange | port status | Green ON | The link is properly connected (link up) 10G and action |
|  |  |  | Orange ON | The link is properly connected (link up) 1G/2.5G and action |
|  |  |  | OFF | The link is interrupted (link down). |
| 40G/100G QSFP28 | Green | Port status | ON | The link is properly connected (link up) and action |
|  |  |  | OFF | The link is interrupted (link down). |

Power Supply and FAN

The power ports are installed on the rear panel of the device. Only one power port is supported.The power ports and fans are fixed in structure.Hot swap is not supported.

| Characteristic | S5863-24H2QC |
| --- | --- |
| Nominal AC/DC power supply | AC: 100～240V; 47/63Hz |
| AC Rated power of power supply | 100W |

| Characteristic | S5863-24H2QC |
| --- | --- |
| FAN | 4*8000 RPM adjustable fan |

Software Specifications

| Attributes | S5863-24H2QC |
| --- | --- |
| VLAN | Support 4K VLAN Support VLAN based on port, MAC and protocol Support dual Tag VLAN, Access/Trunk/Hybrid |
| Ethernet | Ethernet interface operating modes Ethernet interface operating rates Jumbo Frame Port enable/disable Port based storm-control Unknow-unicast/unknow-multicast/broadcast storm-control Port-isolate |
| Stackable | Support Sacking of 8 units through a single ip address. Support ‌Unified management.‌ Support Redundancy mechanism. Support High speed interconnect. |
| VXLAN | Support VXLAN uses MAC in UDP encapsulation Support VMS can move between different Layer 2 network domains. ‌Support VNI is used to isolate different virtual networks to ensure that network traffic of different tenants is isolated ‌. Support Enable virtual machine packets to communicate across different subnets and data centers; ‌ |
| MAC | Support static MAC address setting Support black hole MAC address filtering Support port MAC address limit Automatic learning and aging of MAC addresses |
| ARP | Static and dynamic ARP entries Aging of ARP entries ARP-Proxy |
| Port Control | Support two-way bandwidth control Support port storm suppression Support 9K Jumbo ultra-long frame forwarding |
| Port Aggregation | Support static link aggregation Support dynamic LACP Each aggregation group supports a maximum of 8 ports LAG load balance（SIP/DIP/SMAC/DMAC） |
| Mirroring | Supports multi-port mirroring analysis Service flow-based image analysis |
| ACL | Support standard and extended ACL Support ACL policy based on time period Provide MAC, IP, L4 port and port level security control functions |
| QoS | Traffic classification based on COS/DSCP (simple classification) Traffic classification based on ACL (complex classification) Traffic classification based on inner header of the tunnel packets Queue scheduling Remark the priority fields (COS/DSCP) of the packet based on ACL Flow redirection Flow mirror Traffic policing based on direction(in/out) of Port Traffic policing based on direction(in/out) of VLAN Traffic policing based on direction(in/out) of flow Traffic policing based on direction(in/out) of aggregated flow SP（Strict Priority）scheduling WRR（Weighted Round Robin）scheduling SP + WRR mixed scheduling |
| Safety | Support user hierarchical management and password protection Support IEEE 802.1X authentication Support Radius & TACACS+ authentication Support MAC address learning limit, support black hole MAC function Support port isolation Support broadcast message rate suppression Support IP Source Guard Support ARP flood suppression and ARP spoofing protection |
| Layer 3 | Support ARP learning and aging Support IPv4/IPv6 static route Support dynamic route RIP/OSPF/BGP Support IPv4/IPv6 dual-stack platform Support VRRP、CLI、Telnet、WEB、SNMP V1/V2/V3、SSHv2 mode |
| Terminal Services | Configurations through CLI Console Terminal service |
| Maintenance | Per-module Debug features CPU usage display and alarm Memory usage display and alarm Device temperature、PSU、FAN、status display and alarm User operation logs Management of logs, alarms, and debugging information Detailed Diagnostic-information collection Manual /Schedule reboot Reboot Information logging Ping IPv6 Ping Traceroute Port/Flow/Remote mirror To CPU/From CPU packets statistics Time configuration Timezone |
| System Security | SSHv1/v2 RADIUS TACACS+ Authentication Accounting Port based dot1x MAC based dot1x MAC/IP ACL Port/VLAN/L4-Port ACL IP Source Guard Limitation on MAC address learning on interface Rate limit CPU Traffic Limit CLI/WEB/SNMP/Telnet/SSH filtering |
| Spanning-Tree | Rapid Spanning-Tree Protocol Multi-instance Spanning-Tree Protocol |
| Network Management | DHCP Server DHCP Relay DHCP Snooping DHCP Option60 DHCP Option82 DHCP Option66 DHCP Option120 SNTP Simple Network Time Protocol） LLDP |
| Configuration Management | Inband management interface and configuration Outband management interface and configuration Privileged user priority and privileged commands Network management based on SNMPv1/v2c/v3 Public and private MIB Public and private Trap Configuration and management based on WEB Restore factory default configuration |
| File system | Upload and download files through FTP or TFTP Upload and download files through Xmodem |
| System Management | Support FTP, TFTP file upload and download Support SNTP Support LLDP neighbor device discovery protocol Support RFC1213 SNMP network management Support Ping and Traceroute Supports configuration management based on WEB pages |
| Upgrade | Upgrade with the local image file Upgrade with the remote TFTP server Online upgrade Uboot |

Connector Specifications

The switch provides standardized interfaces, thus facilitating rapid and trouble-free integration into the existing network infrastructures.

Network Interfaces

The switch optical interface ports for SFP/SFP+ modules and below table is available SFP/SFP+ module list.

| Index | SFP/SFP+ Module Description |
| --- | --- |
| 1 | - Wavelength: 850 nm / Distance: 550 m / Mode: multi-mode - Connector: LC / Data rate: 1.25Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 2 | -. Wavelength: 1310nm / Distance: 5km / Mode: single-mode -. Connector: LC / Data rate: 1.25Gbit/s / Core type: Dual Core -. Operating Temperature : 0 °C – 70 °C |
| 3 | - Wavelength: 1310 nm / Distance: 10 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C – 70 °C |
| 4 | - Wavelength: 1310 nm / Distance: 15 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C – 70 °C |
| 5 | - Wavelength: 1310 nm / Distance: 20 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 6 | - Wavelength: 1310 nm / Distance: 40 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 7 | - Wavelength: 850nm / Distance: 300m / Mode: Multimode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C - 10GBASE-SR (10G) |
| 8 | - Wavelength: 850nm / Distance: 300m / Mode: Multimode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C - 1000BASE-SX (1G), 10GBASE-SR (10G） |
| 9 | - Wavelength: 1310nm / Distance: 10Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C - 10GBASE-LR (10G) |
| 10 | - Wavelength: 1550nm / Distance: 40Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C - 10GBASE-ER (10G) |
| 11 | - Wavelength: 1310nm / Distance: 70Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C - 10GBASE-ZR (10G） |
| 12 | - Wavelength: 850nm / Distance: 100m / Mode: MMF - Connector: MMF Data rate: 100 Gbit/s / Core type: Dual Core - Operating Temperature : 0°C ~ 70 °C 100GBASE-SR4 |