S6650-48SX6QC

L3 Managed Ethernet Switch

https://www.nexavm.com

# Product Overview

The S6650-48SX6QC switch is a high-performance 10G fiber switch developed by our company. It features advanced hardware design and a hot-swappable power supply for redundancy. This switch supports up to 48*1G/10G SFP+ ports, along with 6*40GE/100GE QSFP28 uplink ports. It offers full-port L2/L3 wire-speed forwarding, IPv6 support, and a range of business features, including ACL policies, and network security.

The S6650-48SX6QC provides the highest 10 Gigabit port density and maximum switching capacity in its class, supporting wire-speed forwarding on all ports. It also accommodates flexible access for both 10GE and 100GE connections and automatically recognizes the type of optical module installed, maximizing user investment protection and ensuring flexibility.

Additionally, designed for high-density and high-bandwidth applications, this switch meets the demands of next-generation enterprise networks, data centers, and metropolitan area networks. It is suitable for network operators, campus Ethernet aggregation or access layers, data center access layers, and small to medium-sized enterprise core layers, offering comprehensive server access solutions for data centers.

# Device Brief Information

|  | S6650-48SX6QC |
| --- | --- |
|  | - 48*1GE/10GE SFP+ - 6*40GE/100GE QSFP28 - 2160Gbps Switching capacity - Size: 440×410×44mm 19” Rack-mount - AC+AC (or AC+DC，or DC+DC) power supply - AC: 100~240V; 47/63Hz - DC: -20V~-60V - 2*Modular FAN supply |

Key Features

Equipment Stability

- Support IPv4/IPv6 dual protocol stack platform based on Linux operating system

- Support multiple link redundancy and network redundancy protocols such as STP / RSTP / MSTP / ERPS / LACP

Carrier-level equipment stability

- Based on Linux operating system, support IPv4/IPv6 dual protocol stack platform

- Support multiple link redundancy and network redundancy protocols such as STP/RSTP/MSTP/ERPS/LACP

Stackable

- Support Combine the processing power of multiple switches into a single logical device.

- Support When a switch in the stack system fails, traffic can be automatically switched to other switches, network availability and redundancy.

- Support Stacking simplifies the management process. Administrators only need to manage one logical device.

Perfect safety mechanism

- Support ACL security filtering mechanism, which can provide security control functions based on MAC, IP, L4 port

- Support ARP attack automatic protection and user blocking functions

- Support DHCP attack automatic protection based on Mac Address and user blocking functions

- Support DHCP-snooping/IP-source-Guard/802.1X and other security features

- Support link protection functions such as BFD, FlexLink, LACP

- Support remote loop detection function

- Support remote user authentication based on Tacacs+, Radius, and Local user authentication

Powerful QoS capabilities

- Support port speed and flow speed limit function

- Support classification based on service flow and Qos flow control function

- Support queue scheduling algorithms such as SP/WRR/SP+WRR

- Support port-based and service flow-based mirroring functions

Enhanced multi-service capabilities

- Support DHCP Server and DHCP Relay

Rich routing protocol

- Support static route

- Support dynamic route such as RIP, OSPF, BGP,

- Support IPV4/IPV6 dual protocol stack, support RIPng、OSPFv3

Simple and easy-to-use network management function

- Support CLI based on RS232 serial port, Telnet and SSHv2

- Support WEB-based configure operation management, support SNMP V1/V2/V3

- Support remote upgrade or equipment through FTP and TFTP

Hardware Specifications

| Attributes | S6650-48SX6QC |
| --- | --- |
| Port | 48*1GE/10GE SFP+ 6*40GE/100GE QSFP28 |
| Chip Solution | 98DX8548 (Marvell) |
| Exchange Capacity | 2160Gbps |
| Packet Forwarding Rate | 600Mpps |
| Memory and Storage | Memory: 4GB; Storage: 8GB |
| MAC Address | 32K |
| Packet Buffer | 48Mbit |
| Power | AC: 100～240V; 47/63Hz; DC: -20V ~ -60V |
| Power Consumption | ≤220W |
| Weight | ≤9kg |
| Dimensions (W*H* D) | 440×410×44mm (L*W*H) |
| Environment | Working Temperature: -10℃～50℃ Storge Temperature: -40℃～70℃ Relative Humidity :5%~95%, non-condensing |

Panel view

| Label | Connector Type | Connector Description |
| --- | --- | --- |
| 1G/10GE 1-48 | SFP+ | Connect the user port |
| 40G/100G 1-6 | QSFP28 | Connect to external uplink equipment |
| USB | USB2.0/3.2 | Connecting peripherals for uploading and downloading configuration |
| CONSOLE | RJ45 | The software debug port |
| MGMT | RJ45 | The out-band management port |

LED indicator

| LEDs/Buttons | Color | Function | Description |  |
| --- | --- | --- | --- | --- |
| PWR1(left) PWR2(right) | Green | Power status | ON | The AC/DC power module is normal. |
|  |  |  | OFF | The AC/DC power module faulty or unavailable |
| FAN1(left) FAN2(right) | Green | Fan running status | ON | FAN is running normally. |
|  |  |  | OFF | FAN is running abnormally. |
| SYS | Green | System running status | Green OFF | The device is not running or power off. |
|  |  |  | Green Flash | The device is running normally. |
| USB | Green | Usb flash insertion status | ON | Insert flash (Fat32) |
|  |  |  | OFF | NO Insert flash (Fat32) |
| MGMT | Green | Port status | ON | The link is properly connected (link up) and in action |
|  |  |  | OFF | The link is interrupted (link down). |
| STK | Green | Port status | ON | Enable Stack |
|  |  |  | OFF | Disable Stack |
| QSFP28 Port | Green | Port status | ON/Flash | The link is properly connected (link up) and in action Works in 10G/100G mode. |
|  |  |  | OFF | The link is interrupted (link down). |
|  | Orange |  | ON/Flash | The link is properly connected (link up) and in action Works in 100/1000Mbps or 40Gbps mode. |

Plug-In Units

Power Supply Unit (PSU)

On the rear panel of the switch, there are two slots for Power Supply Unit (PSU).  The mounting slots can be equipped with DC or AC type PSUs.  The different type of PSUs can be equipped into the switch.  The below shows AC and DC type of power supply modules Two PSUs, for supplying redundancy, dual PSUs have to be installed at a system.

The power supply unit (PSU) feeds the proper power voltage to the base board of switch.  Two PSUs are possible to be mounted into rear side of the chassis and it can be DC or AC type of PSU.  Therefore, you need to decide to choose proper power supply unit according to installation environment.

The power connector (AC and DC type) for main power supply is located on the rear panel of the PSU.  For redundant power feeding, the second PSU has to be used.

The below shows the electrical characteristics of the switch：

| Characteristic | S6650-48SX6QC |
| --- | --- |
| Nominal DC power supply | DC: -20V ~ -60V |
| Nominal AC power supply | 100～240V; 47/63Hz |
| FAN | The temperature starts at 0 degrees and rotates at 30% speed (about 3900 RPM), and the speed increases by 0.78%x13000 rpm for every 1 degree increase. |
| AC Rated power of power supply | 280W |
| DC Rated power of power supply | 220W |

FAN Unit

The switch provides 2 fan unit slots on the rear panel. This plug-in unit internally consists of 2 fans. User can insert fan unit to a fan slot by slowly adjusting them to fit into the slot guid-ance. The air flows from the front side inlet to the rear side outlet.

| Characteristic | S6650-48SX6QC |
| --- | --- |
| FAN | The temperature of the switch chip attachment starts at 30 degrees and rotates at 30% speed (about 3600 RPM). For every 1 degree increase, the speed increases by 1.57%x12000 rpm. |

Software Specifications

| Attributes | S6650-48SX6QC |
| --- | --- |
| VLAN | Support 4K VLAN Support VLAN based on port, MAC and protocol Support dual Tag VLAN, Access/Trunk/Hybrid |
| Ethernet | Ethernet interface operating modes Ethernet interface operating rates Jumbo Frame Port enable/disable Port based storm-control Unknow-unicast/unknow-multicast/broadcast storm-control Port-isolate |
| Stackable | Support Sacking of 8 units through a single ip address. Support ‌Unified management.‌ Support Redundancy mechanism. Support High speed interconnect. |
| VXLAN | Support VXLAN uses MAC in UDP encapsulation Support VMS can move between different Layer 2 network domains. ‌Support VNI is used to isolate different virtual networks to ensure that network traffic of different tenants is isolated ‌. Support Enable virtual machine packets to communicate across different subnets and data centers; ‌ |
| MAC | Support static MAC address setting Support black hole MAC address filtering Support port MAC address limit Automatic learning and aging of MAC addresses |
| ARP | Static and dynamic ARP entries Aging of ARP entries ARP-Proxy |
| Port Control | Support two-way bandwidth control Support port storm suppression Support 9K Jumbo ultra-long frame forwarding |
| Port Aggregation | Support static link aggregation Support dynamic LACP Each aggregation group supports a maximum of 8 ports LAG load balance（SIP/DIP/SMAC/DMAC） |
| Mirroring | Supports multi-port mirroring analysis Service flow-based image analysis |
| ACL | Support standard and extended ACL Support ACL policy based on time period Provide MAC, IP, L4 port and port level security control functions |
| QoS | Traffic classification based on COS/DSCP (simple classification) Traffic classification based on ACL (complex classification) Traffic classification based on inner header of the tunnel packets Queue scheduling Remark the priority fields (COS/DSCP) of the packet based on ACL Traffic policing based on direction(in/out) of Port Traffic policing based on direction(in/out) of VLAN Traffic policing based on direction(in/out) of flow Traffic policing based on direction(in/out) of aggregated flow SP（Strict Priority）scheduling WRR（Weighted Round Robin）scheduling SP + WRR mixed scheduling |
| Safety | Support user hierarchical management and password protection Support Radius & TACACS+ authentication Support MAC address learning limit, support black hole MAC function Support port isolation Support broadcast message rate suppression Support IP Source Guard Support ARP flood suppression and ARP spoofing protection |
| Layer 3 | Support ARP learning and aging Support IPv4/IPv6 static route Support dynamic route RIP/OSPF/BGP Support IPv4/IPv6 dual-stack platform Support VRRP、CLI、Telnet、WEB、SNMP V1/V2/V3、SSHv2 mode |
| Terminal Services | Configurations through CLI Console Terminal service |
| Maintenance | Per-module Debug features CPU usage display and alarm Memory usage display and alarm Device temperature、PSU、FAN、status display and alarm User operation logs Management of logs, alarms, and debugging information Detailed Diagnostic-information collection Manual /Schedule reboot Reboot Information logging Ping IPv6 Ping Traceroute Time configuration Timezone |
| System Security | SSHv1/v2 RADIUS TACACS+ Authentication Accounting MAC/IP ACL Port/VLAN/L4-Port ACL IP Source Guard Limitation on MAC address learning on interface Rate limit CPU Traffic Limit CLI/WEB/SNMP/Telnet/SSH filtering |
| Spanning-Tree | Rapid Spanning-Tree Protocol Multi-instance Spanning-Tree Protocol |
| Network Management | DHCP Server DHCP Relay DHCP Snooping DHCP Option60 DHCP Option82 DHCP Option66 DHCP Option120 SNTP Simple Network Time Protocol） LLDP |
| Configuration Management | Inband management interface and configuration Outband management interface and configuration Privileged user priority and privileged commands Network management based on SNMPv1/v2c/v3 Public and private MIB Public and private Trap Configuration and management based on WEB Restore factory default configuration |
| File system | Upload and download files through FTP or TFTP Upload and download files through Xmodem |
| System Management | Support FTP, TFTP file upload and download Support SNTP Support LLDP neighbor device discovery protocol Support RFC1213 SNMP network management Support Ping and Traceroute Supports configuration management based on WEB pages |
| Upgrade | Upgrade with the local image file Upgrade with the remote TFTP server Online upgrade Uboot |

Connector Specifications

The switch provides standardized interfaces, thus facilitating rapid and trouble-free integration into the existing network infrastructures.

Network Interfaces

The switch optical interface ports for SFP/SFP+ modules and below table is available SFP/SFP+ module list.

| Index | SFP/SFP+/QSFP28 Module Description |
| --- | --- |
| 1 | - Wavelength: 850 nm / Distance: 550 m / Mode: multi-mode - Connector: LC / Data rate: 1.25Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 2 | -. Wavelength: 1310nm / Distance: 5km / Mode: single-mode -. Connector: LC / Data rate: 1.25Gbit/s / Core type: Dual Core -. Operating Temperature: 0 °C – 70 °C |
| 3 | - Wavelength: 1310 nm / Distance: 10 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C – 70 °C |
| 4 | - Wavelength: 1310 nm / Distance: 15 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C – 70 °C |
| 5 | - Wavelength: 1310 nm / Distance: 20 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 6 | - Wavelength: 1310 nm / Distance: 40 km / Mode: single-mode - Connector: LC / Data rate: 1.25 Gbit/s / Core type: Dual Core - Operating Temperature: 0 °C - 70 °C |
| 7 | - Wavelength: 850nm / Distance: 300m / Mode: Multimode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C - 10GBASE-SR (10G) |
| 8 | - Wavelength: 850nm / Distance: 300m / Mode: Multimode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C - 1000BASE-SX (1G), 10GBASE-SR (10G） |
| 9 | - Wavelength: 1310nm / Distance: 10Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C - 10GBASE-LR (10G) |
| 10 | - Wavelength: 1550nm / Distance: 40Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C - 10GBASE-ER (10G) |
| 11 | - Wavelength: 1310nm / Distance: 70Km / Mode: Single mode - Connector: LC / Data rate: 10.3125 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C - 10GBASE-ZR (10G） |
| 12 | - Wavelength: 850nm / Distance: 100m / Mode: MMF - Connector: MMF Data rate: 100 Gbit/s / Core type: Dual Core - Operating Temperature: 0°C ~ 70 °C 100GBASE-SR4 |

Purchase Info

| Product Name | Product Description |
| --- | --- |
| S6650-48SX6QC | 48*1GE/10GE SFP+ & 6*40GE/100GE QSFP28；Power module Optional； Support Dual-FRU AC or DC redundant power supply； |